Shutter measuring sheet

Measure every window before your survey, so you can compare quotes properly

How to measure

1. Use a steel tape and measure in millimetres. Decide if the shutter will sit inside the recess (most common) or on the wall outside it.

2. Width: measure the inside of the recess at the top, middle and bottom. Write down all three.

3. Height: measure at the left, middle and right, from the top of the recess to the sill.

4. Use the smallest width and smallest height for an inside-recess fit. Recesses are rarely perfectly square.

5. Depth: measure how deep the recess is, and how far window handles stick out. Shutter panels need room to open.

6. Note anything in the way: handles, radiators, sockets, tiled sills, curtain poles, alarm sensors.

Measure the inside of the recess at three points for width (top, middle, bottom) and three for height (left, middle, right). Use the smallest of each.

Your windows

| Room / window | Inside or outside | W top | W mid | W bottom | H left | H mid | H right | Depth | Handle | Style wanted |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Obstructions and notes

Bay windows: one line per section

Number the sections left to right. Measure each one separately and note the angle where it meets the next section (e.g. 135° for a typical angled bay, 90° for a square bay).

| Section | W top | W mid | W bottom | H left | H mid | H right | Angle to next | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Sketch the bay from above (number each section)

Questions to ask each installer

☐  Is the price supplied and fitted, including VAT?

☐  Which material (MDF, faux wood, hardwood) and why for this room?

☐  Will you re-measure before manufacture, and who pays if the size is wrong?

☐  How long is the lead time, and what is the guarantee?

☐  Do bay windows need extra frames, trims or battens?

| Quote 1 (company and price) | Quote 2 (company and price) | Quote 3 (company and price) |
| --- | --- | --- |